2022-02-09

| # What is this experiment for? | # Triplets v3: pink tip – fast velocities |
| --- | --- |

# Checklist

☒Position Tiles

☒Turn Vacuum off

☒Put measurement cap on

☒Turn Camera light off

☒Measure tiles

☒Put cap off

☒Turn Vacuum on

☐Glue:

| Start Time: | 13:30 |  |  |
| --- | --- | --- | --- |
|  | Measurements / Parameters to test | Stopwatch time | Comments |
| Glue Temperature: | 50°C |  |  |
| Weight Glue: | 10.019g |  |  |
| Weight Hardener: | 0.759 g | Start Stopwatch |  |
| Glue Temperature: | 27 °C | 00 |  |
| Degas Pictures taken after X min: | 5, 8:30, 10:00 min | 00 |  |
| Glue Temperature: | 28 °C | 00 |  |
| Filling Syringe |  | 12:00 |  |
| Testing Dispenser |  | 14:00 |  |
| Leveling Procedure |  | 00 |  |
| Start Triplet 1 | V=10 | 19:00 | One edge failed -> bad hight calibration. New calibration and it worked |
| End Triplet 1 |  | 21:30 |  |
| Start Triplet 2 | V=12.5 | 22:25 | Brown triplet |
| End Triplet 2 |  | 23:00 |  |
| Start Triplet 3 | V=15 | 23:40 | Yellow triplet |
| End Triplet 3 |  | 24:00 |  |
| Turn Vacuum off |  | 24:40 |  |
| Put measurement cap on |  | 0 |  |
| Start measurement (Repetion: x24) |  | 27:24 |  |

| after 5 min | after 8 min | after 10 min |
| --- | --- | --- |

## Results

|  | Triplet Description - Blue Triplet - v = 10 mm/s - ϕ=0.021cc/s Vac release right after gluing |
| --- | --- |
|  | Triplet Description - brown Triplet - v = 12.5 mm/s - ϕ=0.021cc/s Vac release right after gluing |
|  | Triplet Description - yellow Triplet - v = 15 mm/s - ϕ=0.021cc/s Vac release right after gluing |

## Analysis Results: